Horst F.'s

IDE-Harddisk-Power-Management

SB

V1.3 1.8.1993

(V1.4 10.8.1995)

What it does

------------

The program was created to switch off IDE-HDs by software immediately

or after a selectable time. They will switch on by themselfs on the

next operation. That is useful for saving current and decreasing

noise. You will like it if you leave your PC permanently switched on.

For demonstration of saving current two examples:

Mode            current consumption in Watt

1.              2.

--------------------------------------------------

ACTIVE          5.0             1.9

IDLE            2.75            0.8

STANDBY         0.66            0.25

START           7.0             3.9

The last line shows, that your auto-powerdown time should be big

enough to avoid a high current consumption.

What does it need

-----------------

The program needs at first IDE-HDs with a so called power management

(PM). This equipment was developed especially for laptops and

notebooks for saving accumulator capacity. Fortunately it seems that

for some producers it's to expensive to make HDs with and without PM.

And so they make HDs only with PM. Information about if a HD has PM

possibly can give the dealer. If he can't, try to call the producer

(if you have a phone number) or write him (if you have the adress).

The last possibility is to try out my program. Owner of HDs by Maxtor

or Conner have good cards. Also Quantum and Western Digital make most

of their HDs with PM. Not so good it's with HDs by Fujitsu and

Seagate, except the ST3283A and ST9xxxA-series.

Following HDs has been tested and have PM:

Maxtor          7080A, 7120A

Quantum         LPS120A, LPS240A

Conner          einige

Wester Digital  einige

Tested means in this case, that all tested examples came with PM.

But there can exist also units without.

not tested, but according to documentation with PM:

Seagate         ST3283A

(Product Manual, but HD has no Auto-Standby-Timer

and can only switch to standby modus immediate)

ST9xxxA

(Product Manual)

Seagate         ST3096A/3120A/3144A

(Installation Guide, HDs with ident no.

91100x-0xx, 91100x-1xx, 91100x-4xx)

Fujitsu         M2637, M2618

(Pocket Guide)

Conner          all new one (what ever this means)

(Statement of an employee of Conner)

Following HDs has been tested and have no PM:

Seagate         ST157A

Seagate         ST3144A (see above)

not tested, but according to documentation without PM:

Seagate         ST3096A/3120A/3144A

(Installation Guide HDs with ident no.

91100x-3xx without PM)

Please send me informations about positive or negative tests, so I can

expand the list in the next versions.

What is the way to use it

-------------------------

At first some words about PM: The PM uses different modes with

different consumption of current. These modes are with decreasing

current consumption in this order ACTIV, IDLE, STANDBY and SLEEP.

ACTIVE is the normal mode of a HD. In IDLE mode the drive electronic

ist switched off. In addition the drive motor is switched off in

STANDBY mode. If the interface electronic also is off the drive is in

SLEEP mode. This mode can be left only with a drive reset. Some times

the SLEEP mode is not implemented or a synonym for STANDBY. In addition

normally exists a so called auto-standby. With this mechanism the

drive switchs to STANDBY after a time (auto-standby timer) by itself.

Some drives also have an auto-idle timer.

How to call my program:

SB [A] [F] [t1|-|+|cmd1] [A] [F] [t2|-|+] [cmd2]

|               | |                 |

+- for drive 1 -+ +-  for drive 2  --

A - second IDE command set for the specified drive

F - send command to HD, even if HD not found in CMOS (force)

t - auto-standby time in minutes (0 - 21)

(0 = no auto-standby, without: no change)

- - no changes for the specified drive

+ - standard for the specified drive: no t/(cmd1)

cmd - command : IDLE / ID (electronic off)

STANDBY / SB (electronic and

motor off)

SET / ! (set auto-standby time)

CHECK / ? (check mode)

START / GO (ACTIV mode)

(without: STANDBY)

with two drives: without cmd1, the cmd2 is for both; without t2 and

with t1, the t1 is for both

Along with "IDLE", "STANDBY and "SET" you can use a auto-standby time.

"SET" calls "IDLE" or "STANDBY" according to the actual drive mode. If

you use "GO" with a time value of 0 SB starts the drive without

waiting for completion of the start. This works not for starting two

drives at the same time. The command "CHECK" returns an errorlevel of

0 if tested drive is in STANDBY mode otherwise 255.

example 1:

SB A 10 SET STANDBY

or (short):

SB A 10 ! SB

The auto-standby time of the first drive is set to 10 minutes, the

second IDE command set is used. The second drive goes standby

immediatly (first IDE command set).

example 2:

SB 5 STANDBY

or (short):

SB 5

Does the STANDBY command for both drives (or the only one). The

auto-standby time is set to 5 minutes.

The problem is the right use. Unfortunately the use of PM is only a

half standard. The IDE commands are standardized, indeed. But some

producers read it in different ways or added some own commands. The

program uses only the standardized commands, but there are two

different. With the switch "A" can be switched from the normally used

one to the second if the first should not work. The use of the

auto-standby timer is a little bit complicated. Without mentioning

that some drives haven't an auto-standby it works some times

different. Here some hints to find the right way of use:

- The 7080A and 7120A by Maxtor work like following:

It is possible to set the timer in ACTIVE/IDLE and in STANBY mode.

"SET" sets the timer to the specified time and don't change the

actual mode. With the "STANDBY" command the drive goes standby

immediately in every case. If a time is specified the timer is set.

"IDLE" starts the drive motor.

- The Quantum LPS120A and LPS240A allow to set the timer only in

ACTIVE/IDLE mode but not in STANDBY mode. So the "SET" command

doesn't work right. With the "STANDBY" command the drive goes

standby immediately only if no time is specified, otherwise only the

timer will be set. So for setting the timer the command "STANDBY"

with a time specified has to use in ACTIVE/IDLE mode. The command

"IDLE" doesn't start the drive motor.

If a drive doesn't understand a command, so program will notify you.

The best way to find out if a drive has PM is to use the command

"CHECK" for this drive.

Where are problems

------------------

Without mentioned that there are many problems in the world I will say

some words about possible problems with my program. At first it's a

problem that there is no full standard. May be some commands work not

right or not at all. There could be problems with two drives

(master/slave). So, I can't give guaranty for a right work. The same

applies to any damage of the hardware. But don't be to much afraid,

I've never been heard about such terrible things in connection with my

program, yet. If this should ever happens mail me immediately with the

name of the HD.

What's nice

-----------

At first my program SB. And also good is that SB is Freeware. So, give

it every people you want but only with documentation!

What is history

---------------

V1.0  - fist usable version in assembler

V1.1  - changed to C (Borland C 3.1)

- commands SET, CHECK and START

V1.2  - second command set with switch A

V1.3  - easier parameters

- automatical use of CMOS drive count

V1.4  - switch 'F' added

What is future

--------------

- ".INI" file for standard settings

- use of drive type for SET

- use of special possibilities of some drives

- I don't know

- Abolition of SB, because IDE drives no longer exist

- Abolition of forgetfulness ... hmm, where did I leave off?

What else

---------

Perhaps a cache program programmer can pick up courage to use the

standby feature, so I don't have to wait on write for starting the

drive. My adress - see below.

Who had a shot at this program

------------------------------

Suggestions please send to Horst F.

e-mail: horstf@infaut.et.uni-magdeburg.de