AMD

Frequently Asked Questions

and

Answers

AMD PCnet™-Mobile 802.11 Wireless LAN Drivers

Release 1.5

Document Rev. 1.1

Oct 30, 1997

(This page intentionally left blank)

Frequently Asked Questions

AMD PCnet™-Mobile

Wireless LAN Drivers

# Introduction

This document lists Frequently Asked Questions and Answers that relate to the installation and operational aspects of AMD’s PCnet™-Mobile  family of drivers.  The document is organized on the basis of the different operating system environments that the PCnet™-Mobile family of drivers support.

This is the second release of this document and it includes Frequently Asked Questions and Answers that are applicable to PCnet™-Mobile drivers release version 1.5.

The purpose of this document is to enable OEM/LT and other customers, as well as AMD’s network specialists to quickly resolve frequently encountered issues.  If this document does not address certain questions,  it is recommended that those questions be forwarded to the appropriate AMD support channel.

# Windows 95 / Windows NT

Q1. How do I view the driver error messages and also possible solutions ?

Please refer to ‘Section 7 Appendix B: Driver Error Messages’ in the Manual.doc file (PCnet-Mobile Software Manual). It has information regarding the driver error messages, possible solutions  and also how to view the error messages for Windows95 and WindowsNT.

Q1. Windows 95 fails to recognize the PCnet™-Mobile PCMCIA card.  What can I do to resolve this problem?

Answer :

You may try one or more of the following options:

Make sure that the PCnet™-Mobile card is in a PCMCIA slot during boot.  Remove and reinsert the card if necessary and reboot the system.

In Windows 95, Card and Socket services sometimes fail to initialize the card on the very first load.  Please try to take the card out and re-insert it.  That will cause the driver to reload. Alternatively, you can reboot the machine.

Q2.  When loading the PCnet™-Mobile NDIS 3 driver in Windows 95, I see an error that indicates that the driver in encountering a "24K Memory Window not allocated". What can I do to resolve this problem?

Answer:

You can change BIOS configuration by going to Start->Settings->Control Panel->System

->Device Manager-> System Devices->System Board extension for PnP BIOS->Resources.

Un-check 'Use automatic settings' checkbox and hit 'Change Settings' button.

That will free some memory in 640k-1M range and the driver will be able to load.

Please refer to Appendix B, Section 7.1 in Manual.doc file for more information.

Q3. I am unable to install the NDIS3 miniport driver on ISA PnP adpater cards in Windows NT4.0 systems. The system reports conflict of resources. How do I resolve it?

Answer:

For NDIS3 driver in Release 1.5 we do not support ISA cards on Windows

NT4.0 systems without PnP BIOS present (since Win NT 4.0 is not PnP OS and

relies on the BIOS for PnP device configuration).

To insure that PnP BIOS is present and that PCNet-Mobile is configured

correctly, please run Microsoft utility isolate.exe from DOS. Use

obtained config parameters for PCNet-Mobile in the NDIS3 driver properties

window when you configure NT network.

Q4. The NDIS3 driver for ISA PnP card on Windows NT does not load properly. Any suggestions?

Answer:

You could try the following options -

Please make sure that you uncheck the "PCMCIA card" checkbox in the

ISA WLAN adapter properties.

On Windows NT 4.0 systems you could use the Event Viewer to see the

NDIS3 driver error messages.

Please refer to section 5.1’Installing The Ndis 3.x Miniport Driver’  in the Manual.doc for 	additional information. Also please refer to Appendix B section 7.1 in the Manual.doc file

for additional infornation on driver error messages and possible solutions.

Q5.  Will the PCnet™-Mobile NDIS 3 based solution pass the Microsoft NDIS Certification tests?

Answer:

A number of Microsoft certification tests (NDISTEST) require a hundred percent packet delivery rate in order to pass.  Because of the complexity of the wireless LAN test environment (operating over significantly less reliable media, dynamic topologies, media is unprotected from outside signals), coupled with radio limitations and high error rates, it is impossible to ensure a hundred percent packet delivery rate.  As a result of this, the NDIS 3.x driver and the AMD/Harris/Celestica PCMCIA adapter fail the Microsoft certification test suite (NDISTEST).

# DOS / Windows For Workgroups

Q1. When a driver error message is displayed, how do I resolve it ?

Please refer to ‘Section 7.2 Appendix B: Ndis 2.0.1Driver Error Messages’ in the Manual.doc file. 	It has  information regarding the driver error messages and possible solutions .

Q2.  When trying to  load the PCnet™-Mobile NDIS 2 driver in DOS / Windows For Workgroups, I see an error that indicates that the driver in encountering a "Memory Window Error". What can I do to resolve this problem?

Answer:

The PCnet™-Mobile NDIS 2 device driver requires a 24K memory window in order to successfully load and run.  You are seeing this error because your system is unable to allocate the necessary memory window to the driver.

Most PCMCIA Card and Socket Services providers provide a means such as a configuration utility to  block out a range of memory for use by clients such as the PCnet™-Mobile NDIS 2 driver.  If you have such an utility, you may use it to block out a range of memory for the PCnet™-Mobile NDIS 2 driver.

If the PCMCIA device drivers cannot allocate the memory window, it may help to exclude a block from EMM386 if it is used.  An example of this is:

DEVICE=C:\DOS\EMM386.EXE X=D000-D7FF

Q3. When trying to  load th e PCnet™-Mobile NDIS 2 driver in DOS / Windows For Workgroups for the PCnet™-Mobile PCMCIA card and reboot the machine,  the driver is not loaded. How do I

resolve this issue?

Answer:

Windows for WorkGroup does not support PCMCIA card so before you install the Pcnet™-	Mobile NDIS 2 driver check that the PCMCIA service software is installed. To get the software

you could contact the Laptop vendor or the vendors web site or get it from a third party.

Q4.  When trying to  load the PCnet™-Mobile NDIS 2 driver in DOS / Windows For Workgroups   for ISA PnP card. Then machine is rebooted but the driver is not loaded. How do I resolve this issue?

Answer:

Since Windows for Workgroup does not support PnP you need to verify the IRQ and I/O values do 	not conflict with other devices.

# General PCnet™-Mobile Questions

Q1.  I am trying to connect the PCnet™-Mobile platform to a non AMD IEEE 802.11 based wireless LAN solution.  I am failing to connect the two machines.  How can I resolve this?

Answer:

AMD’s PCnet™-Mobile Release 1.0 supports the IEEE 802.11specification.  However, interoperability with other IEEE 802.11 WLAN solutions cannot be guaranteed at this time.  AMD will address interoperability with other IEEE 802.11 WLAN solutions in future PCnet™-Mobile  releases.